# POS PRINTER SDK

POS PRINTER SDK	1

1.Introduction	3

2.Version release record	4

3.Function reference	4

public static void POS_Open(string sPortName, int nComBaudrate, int nComDataBits, int nComStopBits, int nComParity, int nParam)	4

public static Boolean POS_IsOpen()	5

public static void POS_Close()	5

public static void POS_SetWriteTimeout(int timeout)	5

public static void POS_SetReadTimeout(int timeout)	5

public static void POS_Write(byte[] buffer)	6

public static void POS_Read(byte[] buffer)	6

public static void POS_Reset()	6

public static void POS_SetMode(int nPrintMode)	6

public static void POS_SetMotionUnit(int nHorizontalMU, int nVerticalMu)	7

public static void POS_SetCharSetAndCodePage(int nCharSet, int nCodePage)	7

public static void POS_FeedLine()	9

public static void POS_SetLineSpacing(int nDistance)	9

public static void POS_SetRightSpacing(int nDistance)	9

public static void POS_CutPaper(int nMode, int nDistance)	10

public static void POS_RTQueryStatus(byte[] buffer)	11

public static void POS_S_SetAreaWidth(int nWidth)	13

public static void POS_S_TextOut(string pszString, int nOrgx, int nWidthTimes, int nHeightTimes, int nFontType, int nFontStyle)	13

public static void POS_S_SetBarcode(string pszInfoBuffer, int nOrgx, int nType, int nWidthX, int nHeight, int nHriFontType, int nHriFontPosition)	15

public static void POS_PrintBitmap(Bitmap orgBitmap)	17

public static void POS_PL_SetArea(int nOrgx, int nOrgy, int nWidth, int nHeight, int nDirection)	18

public static void POS_PL_TextOut(string pszString, int nOrgx, int nOrgy, int nWidthTimes, int nHeightTimes, int nFontType, int nFontStyle)	19

public static void POS_PL_SetBarcode(string pszInfoBuffer, int nOrgx, int nOrgy, int nType, int nWidthX, int nHeight, int nHriFontType, int nHriFontPosition)	21

public static void POS_PL_Print()	23

public static void POS_PL_Clear()	24

4. Appendix ………………………………………………………………………………………………………..24

Appendix A  Printer supplementary instruction	24

Appendix B  Barcode instruction…………………………………………………………………………..26

Appendix C  USB interface instruction	32

# 1.Introduction

SerialPosLib.dll is a C# dynamic depot based on vs2005, to conveniently directly control the printer by program. Its functions mainly include the following 3 parts:

1．General function

This function supports printer’s 3 types printing modes simultaneously (standard mode, page mode, label mode).

| POS_Open | Open the interface, then set up the tele-communication with printer |
| --- | --- |
| POS_Close | Close the interface，disconnect with printer |
| POS_Reset | Reset the printer |
| POS_SetMode | Set the printing mode and paper out model |
| POS_SetMotionUnit | Set the motion unit of printer |
| POS_SetCharSetAndCodePage | Set Character sets and codepage |
| POS_FeedLine | Set linefeed |
| POS_SetLineSpacing | Set line spacing |
| POS_SetRightSpacing | Set right spacing |
| POS_QueryStatus | Query status |
| POS_RTQueryStatus | RT query status |
| POS_KickOutDrawer | Kick out drawer |
| POS_CutPaper | Cut paper |

2．Function of supporting standard printing mode (line mode) only.

| POS_S_SetAreaWidth | Set area width |
| --- | --- |
| POS_S_TextOut | Set text out |
| POS_S_SetBarcode | Set barcode printing |

3. Function of supporting page mode printing (P) or label mode printing (L).

| POS_PL_SetArea | Set area |
| --- | --- |
| POS_PL_TextOut | Set text out |
| POS_PL_SetBarcode | Set barcode |
| POS_PL_Print | Printing full page or label |
| POS_PL_Clear | Clear page or label data buffering |

# 2.Version release record

Version: 2.0.1

Date:  13rd August, 2013

Remark:

Official version first released.

# 3. Function reference

# Public static void POS_Open(string sPortName, int nComBaudrate, int nComDataBits, int nComStopBits, int nComParity, int nParam)

Description:

Open the designated serial and Ethernet ports parameters:

sPortName  example: "COM1","COM2"...

nComBaudrate 9600, 19200, 38400, 57600, 115200 Baudrate

nComDataBits 5,6,7,8

nComStopBits 0,1,2  successively indicates stop bit 1,1.5,2

nComParity 0,1,2,3,4   successively indicates parity bit None,Odd,Even,Mark,Space

nParam 0 indicates flow control off,  “0x1”  indicates DTR/RTS flow control控on.

Remarks:

Whether the ports on/off should be distinguished by POS_IsOpen(). If fail to open the ports, pleaese refer to “lasterror” to find out reasons. Only few functions request to set “lasterror”.

This function can be for open Ethernet port and USB port as well, but only one of these two work at a time. Which port to work depending on “nParam”, if “nParam” says “0 or 1”，then “sPortName” means serial port, example “com1，com2” etc. If “nParam” says “2”, then “sPortName” means Ethernet port, example “192.168.0.67” etc. If “nParam” says “3”，then means printing to driver port, example “RP80 Printer” etc.

## Public static Boolean POS_IsOpen()

Description:

Port to be opened or not.

## Public static void POS_Close()

Description:

Close the ports already on.

## Public static void POS_SetWriteTimeout(int timeout)

Description:

Set input-timeout for serial port, null for USB port.

Parameter:

timeout millisecond.

## Public static void POS_SetReadTimeout(int timeout)

Description:

Set read timeout for serial port, null to USB port.

Parameter:

timeout millisecond.

## Public static void POS_Write(byte[] buffer)

Description:

If serial port already on, then data of buffer will be sent out to serial port.

Parameter:

buffer byte array needed to be sent out.

## Public static void POS_Read(byte[] buffer)

Description:

If serial port opened /on, then data will be read from serial port to fill buffer, null to USB port.

Parameter:

buffer receives data buffering area.

## Public static void POS_Reset()

Description:

Reset printer to clear the printing buffer data, and the on setting of character and line spacing. Then printing mode restores to the default mode.

## Public static void POS_SetMode(int nPrintMode)

Description:

Set the printing mode.

Parameter:

nPrintMode

[in]  designates to printing mode.

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_PRINT_MODE_STANDARD | 0x00 | Standard mode (Line mode) |
| POS_PRINT_MODE_PAGE | 0x01 | Page mode |
| POS_PRINT_MODE_BLACK_MARK_LABEL | 0x02 | Black mark label mode |

## Public static void POS_SetMotionUnit(int nHorizontalMU, int nVerticalMu)

Description:

Set printer’s motion unit.

Parameter:

nHorizontalMU

[in]  set horizonal motion unit to be 25.4 / nHorizontalMU millimeter, 0~255 available.

nVerticalMU

[in]  set vertical motion unit to be 25.4 / nVerticalMU millimeter, 0~255 available.

## Public static void POS_SetCharSetAndCodePage(int nCharSet, int nCodePage)

Description:

Select the character sets and codepage.

Parameter:

nCharSet

[in]  designates character sets. Different character sets have different definitions for ASCII value of “0x23~0x7E”.

Following values available:

| Value | Meaning |
| --- | --- |
| 0x00 | U.S.A |
| 0x01 | France |
| 0x02 | Germany |
| 0x03 | U.K. |
| 0x04 | Denmark I |
| 0x05 | Sweden |
| 0x06 | Italy |
| 0x07 | Spain I |
| 0x08 | Japan |
| 0x09 | Nonway |
| 0x0A | Denmark II |
| 0x0B | Spain II |
| 0x0C | Latin America |
| 0x0D | Korea |

nCodePage

[in]  designates character’s codepage. Different codepages have different definition for ASCII value of “0x80~0xFF”, please refer to printer’s self-testing.

## Public static void POS_FeedLine()

Description:

Paper forward.

## Public static void POS_SetLineSpacing(int nDistance)

Description:

Set character’s line spacing.

Parameter:

nDistance

[in]  Designates line spacing dot value.

0~255 available, distance of every dot concerns to printing head’s resolution.

## Public static void POS_SetRightSpacing(int nDistance)

Description:

Set character’s right spacing (distance of every near character)

Parameter:

nDistance

[in]  designates right spacing dot value.

0~255 available, distance of every dot concerns to printing head’s resolution.

public static void POS_KickOutDrawer(int nID, int nOnTimes, int nOffTimes)

To be confirmed.

## Public static void POS_CutPaper(int nMode, int nDistance)

Description:

Cut paper.

Parameter:

nMode

[in]  Designates the paper cutting mode.

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_CUT_MODE_FULL | 0x00 | Full cut |
| POS_CUT_MODE_PARTIAL | 0x01 | Semi cut |

nDistance

[in]  Designates the dots of paper-in length.

0~255 available, distance of every dot concerns to printing head’s resolution.

Remarks:

1．If designated to be full cut mode, then ignore the parameter “nDistance”.

- If designated to be semi cut mode, then paper slips “nDistance” dots to cut.

### Public static void POS_RTQueryStatus(byte[] buffer)

Description:

Transfer printer status on real time.

Parameter:

buffer receives buffering data area, 4 bytes.

Remarks:

Printer returns to relate status, when receives this command.

Please try not to insert this command into command array with 2 or more bytes.

Printer will do this command immediately when it receives, even though it is doing printing.

Printer will return when gets this command, whatever status it stays.

Return values defined as below:

n=1 correspond    buffer[0]

n=2correspond     buffer[1]

…

## Public static void POS_S_SetAreaWidth(int nWidth)

Description:

Set the printing width under standard printing mode.

Parameter:

nWidth

[in]  Designates the width of printing

0~65535 dots available, but printing width should not exceed 576 limited to the paper width.

## Public static void POS_S_TextOut(string pszString, int nOrgx, int nWidthTimes, int nHeightTimes, int nFontType, int nFontStyle)

Description:

Transfer the character string needed to be printed to buffer area, and given an absolute starting position in X direction (horizon), designates enlarge percentage, types and style for every character in the width and height.

Parameter:

pszString

[in]  the character string needed to be printed.

nOrgx

[in]  Designates the starting position dot from the left edge.

0~65535 available.

nWidthTimes

[in]  Designates the enlarge percentage of character in width.

0~7 available.

nHeightTimes

[in]  Designates the enlarge percentage of character in height.

0~7 available.

nFontType

[in]  Designates the font type for character.

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_TYPE_STANDARD | 0x00 | Standard ASCII |
| POS_FONT_TYPE_COMPRESSED | 0x01 | Compressed ASCII |

nFontStyle

[in]  Designates the character font style.

One or more of following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_STYLE_NORMAL | 0x00 | Normal |
| POS_FONT_STYLE_BOLD | 0x08 | Bold |
| POS_FONT_STYLE_THIN_UNDERLINE | 0x80 | 1 dot underline |
| POS_FONT_STYLE_THICK_UNDERLINE | 0x100 | 2 dots underline |
| POS_FONT_STYLE_UPSIDEDOWN | 0x200 | Upright (First line valid) |
| POS_FONT_STYLE_REVERSE | 0x400 | Reverse (black& white character) |
| POS_FONT_STYLE_CLOCKWISE_90 | 0x1000 | 90 degree in clockwise direction |

## Public static void POS_S_SetBarcode(string pszInfoBuffer, int nOrgx, int nType, int nWidthX, int nHeight, int nHriFontType, int nHriFontPosition)

Description:

Set and print label.

Parameter:

pszInfoBuffer

[in]  Label character string. The allowed area and format concerns to the specific label.

nOrgx

[in]  Designates the label starting position dot from the left edge.

0~65535 available.

nType

[in]  Designates the label type.

Following values available, please also refer to “Appendix B Barcode instruction”.

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_BARCODE_TYPE_UPC_A | 0x41 | UPC-A |
| POS_BARCODE_TYPE_UPC_E | 0x42 | UPC-C |
| POS_BARCODE_TYPE_JAN13 | 0x43 | JAN13(EAN13) |
| POS_BARCODE_TYPE_JAN8 | 0x44 | JAN8(EAN8) |
| POS_BARCODE_TYPE_CODE39 | 0x45 | CODE39 |
| POS_BARCODE_TYPE_ITF | 0x46 | INTERLEAVED 2 OF 5 |
| POS_BARCODE_TYPE_CODEBAR | 0x47 | CODEBAR |
| POS_BARCODE_TYPE_CODE93 | 0x48 | CODE93 |
| POS_BARCODE_TYPE_CODE128 | 0x49 | CODE 128 |

nWidthX

[in]  Designates the elementary width for label.

Following values (n) available:

| n | Single elementary module width （continuous） | Double elementary module width（Scatter） |  |
| --- | --- | --- | --- |
|  |  | Norrow elementary width | Wide elementary width |
| 2 | 0．25mm | 0．25mm | 0．625mm |
| 3 | 0．375mm | 0．375mm | 1．0mm |
| 4 | 0．5mm | 0．5mm | 1．25mm |
| 5 | 0．625mm | 0．625mm | 1．625mm |
| 6 | 0．75mm | 0．75mm | 1．875mm |

nHeight

[in]  Designates the dots for label height.

1~255 available, default to be 162 dots.

nHriFontType

[in]  Designates the font type for character HRI (Human Readable Interpretation).

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_TYPE_STANDARD | 0x00 | Standard ASCII |
| POS_FONT_TYPE_COMPRESSED | 0x01 | Compressed ASCII |

nHriFontPosition

[in]  Designates the position for character HRI (Human Readable Interpretation).

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_HRI_POSITION_NONE | 0x00 | No printing |
| POS_HRI_POSITION_ABOVE | 0x01 | Print above the label only |
| POS_HRI_POSITION_BELOW | 0x02 | Print below the label only |
| POS_HRI_POSITION_BOTH | 0x03 | Print both above and below the label |

## Public static void POS_PrintBitmap(Bitmap orgBitmap)

Description:

Print bitmap

Parameter:

orgBitmap  contains the image data of  Bitmap

Remarks:

Bitmap is .Net format. This function will enlarge Bitmap to be times of min. 8 pixel, for example, enlarge 575 to be 576,383 or enlarge to be 384, then transfer it to be solid color, then transfer to the commands to printer, which can be identified.

## Public static void POS_PL_SetArea(int nOrgx, int nOrgy, int nWidth, int nHeight, int nDirection)

Description:

Set the page printing area.

Parameter:

nOrgx

[in]  Designates given area’s starting position dot in X direction (horizon) from the left edge.

0~ 65535 available.

nOrgy

[in]  Designates given area’s starting position dot in Y direction (vertical) from the top edge (current printing head position)

0~65535 available.

nWidth

[in]  Designates the width for printing area (horizon direction)

0~65535。

nHeight

[in]  Designates the height for printing area (vertical direction)

0~65535。

nDirection

[in]  Designates the direction for printing area (original position)

Following values available:

| Flag | Value | Starting Position |
| --- | --- | --- |
| POS_AREA_LEFT_TO_RIGHT | 0 | Top left |
| POS_AREA_BOTTOM_TO_TOP | 1 | Down left |
| POS_AREA_RIGHT_TO_LEFT | 2 | Down right |
| POS_AREA_TOP_TO_BOTTOM | 3 | Top right |

## Public static void POS_PL_TextOut(string pszString, int nOrgx, int nOrgy, int nWidthTimes, int nHeightTimes, int nFontType, int nFontStyle)

Description:

Transfer the character string needed to be printed to buffer area, and given an absolute starting position in X direction (horizon), designates enlarge percentage, types and style for every character in the width and height.

Parameter:

pszString

[in]  Character string needed to be printed.

nOrgx

[in]  Designates the starting position dot in X direction (horizon) from the left edge.

0~ 65535 available.

nOrgy

[in]  Designates the starting position dot in Y direction (vertical) from the top edge.

0~65535 available.

nWidthTimes

[in]  Designates the enlarge percentage for the character in width.

0~7 available.

nHeightTimes

[in]  Designates the enlarge percentage for the character in height.

0~7 available.

nFontType

[in]  Designates the font type for character.

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_TYPE_STANDARD | 0x00 | Standard ASCII |
| POS_FONT_TYPE_COMPRESSED | 0x01 | Compressed ASCII |

nFontStyle

[in]  Designates the style for character.

One or more of following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_STYLE_NORMAL | 0x00 | Normal |
| POS_FONT_STYLE_BOLD | 0x08 | Bold |
| POS_FONT_STYLE_THIN_UNDERLINE | 0x80 | 1 dot underline |
| POS_FONT_STYLE_THICK_UNDERLINE | 0x100 | 2 dot underline |
| POS_FONT_STYLE_REVERSE | 0x400 | Reverse (black& white character) |

## Public static void POS_PL_SetBarcode(string pszInfoBuffer, int nOrgx, int nOrgy, int nType, int nWidthX, int nHeight, int nHriFontType, int nHriFontPosition)

Description:

Set the label/ barcode.

Parameter:

pszInfoBuffer

[in]  The character string needed to be printed. The allowed area and format concerns to the specific label type.

nOrgx

[in]  Designates the label starting position dot from the left edge.

0~65535 available.

nOrgy

[in]  Designates the label starting position dot from the top edge.

0~65535 available.

nType

[in]  Designates the label type.

Following values available, please also refer to “Appendix B Barcode instruction”.

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_BARCODE_TYPE_UPC_A | 0x41 | UPC-A |
| POS_BARCODE_TYPE_UPC_E | 0x42 | UPC-C |
| POS_BARCODE_TYPE_JAN13 | 0x43 | JAN13(EAN13) |
| POS_BARCODE_TYPE_JAN8 | 0x44 | JAN8(EAN8) |
| POS_BARCODE_TYPE_CODE39 | 0x45 | CODE39 |
| POS_BARCODE_TYPE_ITF | 0x46 | INTERLEAVED 2 OF 5 |
| POS_BARCODE_TYPE_CODEBAR | 0x47 | CODEBAR |
| POS_BARCODE_TYPE_CODE93 | 0x48 | 25 |
| POS_BARCODE_TYPE_CODE128 | 0x49 | CODE 128 |

nWidthX

[in]  Designates the elementary width for label.

Following values (n) available:

| n | Single elementary module width （continuous） | Double elementary module width（Scatter） |  |
| --- | --- | --- | --- |
|  |  | Norrow elementary width | Wide elementary width |
| 2 | 0．25mm | 0．25mm | 0．625mm |
| 3 | 0．375mm | 0．375mm | 1．0mm |
| 4 | 0．5mm | 0．5mm | 1．25mm |
| 5 | 0．625mm | 0．625mm | 1．625mm |
| 6 | 0．75mm | 0．75mm | 1．875mm |

nHeight

[in]  Designates the dots for label height.

1~255 available.

nHriFontType

[in]  Designates the font type for character HRI (Human Readable Interpretation).

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_FONT_TYPE_STANDARD | 0x00 | Standard ASCII |
| POS_FONT_TYPE_COMPRESSED | 0x01 | Compressed ASCII |

nHriFontPosition

[in]  Designates the character position for HRI (Human Readable Interpretation).

Following values available:

| Flag | Value | Meaning |
| --- | --- | --- |
| POS_HRI_POSITION_NONE | 0x00 | No printing |
| POS_HRI_POSITION_ABOVE | 0x01 | Print above label only |
| POS_HRI_POSITION_BELOW | 0x02 | Print below label only |
| POS_HRI_POSITION_BOTH | 0x03 | Print both above and below label |

## Public static void POS_PL_Print()

Description:

Buffer data of printing page or label.

Remarks:

1．If function works, paper in to print content, but the buffer data of page or label will be reserved. This function can be referred to print continuously.

## Public static void POS_PL_Clear()

Description:

Clear the buffer data of printing page and label content.

# Appendix

## Appendix A  Printer supplementary instruction

A．1. Printing mode

Generally, printer works under standard mode and page mode only.

Under standard mode, as long as the printer receives line printing or paper-in command, it will do as command.

For example, when refer to function “POS_S_TextOut” and send character“ABCDEF”, printer will do the character string accord with function then store in buffer area, waiting for printing. When refer to function “POS_FeedLine”, paper feeds and the character string will be printed out. However, if the length exceeds the length of received buffering area, printer will feed paper and print automatically.

Under page mode, printer will store all the received processed data to page buffer area without printing. When refers to function “POS_PL_Print”, it will feed paper and print.

For example, when refer to function “POS_PL_TextOut” and send character string “ABCDEF”, printer will do the character string accord with function then store in buffer area, waiting for printing, if further refers to function “POS_FeedLine”, printer will locate the printing position to the next line head. If refers to function “ POS_PL_Print”, printer will feed paper and print out the data stored in buffer.

Under these two printing modes, data in buffer area will not be cleared automatically. If under page mode, data can be cleared by function “POS_PL_Clear”.

In addition, there is another printing mode call “label mode”, it includes black mark label mode and white mark label mode. Label mode’s printing principle can be refer to page mode.

A．2  Setting printing area

1． Printing area is set by function “POS_PL_SetArea”. If paper feeding and printing commands are finished before referring this function, printer will print content at left side (x0，y0). Width of rectangle printing area (dx dot) expands at Dx (vertical at paper in)  to right side; Height expand down at Y direction (paper in direction). If no refer to this function to set printing area, then printing area will be defaulted.

2． After the printer’s printing area and printing area direction are set up by POS_PL_S etArea), its character, bitmap or down-left label in the printing area will be served as base point.

The downloaded bitmap in printing data and label data take the current location as its down-left, aligning with baseline.

- Before receiving a command including paper-in, if the printing data is already over the printing area (containing character spacing), then printer will auto paper-in forward 1 line (the length paper-in depends on the line-height set by POS_SetLineSpacing) and printing position locates to next line head.

4． The default line-height is 1/6 inch, if the vertical resolution is 203DPI, then approximately to be 34 dots. If the printing characters overlap with the last lines’, caused by: next line containing enlarged characters over 2 times in vertical, or bitmap occupies 2 lines even more, label is higher than normal character and the amount of printer’s paper feeding cannot meet, then by adding line-height will avoid this matter.

## Appendix B  Barcode instruction

B．1  Barcode types

| Barcode types | Allowed length | Character sets |
| --- | --- | --- |
| UPC-A | 11 ≤ n ≤ 12 | 48 ≤ d ≤ 57 |
| UPC-E | 11 ≤ n ≤ 12 | 48 ≤ d ≤ 57 |
| JAN13 (EAN13) | 12 ≤n ≤ 13 | 48 ≤ d ≤ 57 |
| JAN 8 (EAN8) | 7 ≤n ≤ 8 | 48 ≤ d ≤ 57 |
| CODE39 | 1 ≤ n ≤ 255 | 45 ≤ d ≤ 57, 65 ≤ d ≤ 90, 32, 36, 37,43 d1 = dk = 42 |
| ITF | 1 ≤ n≤ 255 | 48 ≤ d ≤ 57 |
| CODABAR | 1 ≤ n≤ 255 | 48 ≤ d ≤ 57 65 ≤ d ≤ 68, 36, 43,45,46,47 58 |
| CODE93 | 1 ≤ n≤ 255 | 0 ≤ d ≤ 127 |
| CODE128 | 2 ≤ n≤ 255 | 0 ≤ d ≤ 127 |

B．2．1  128-code summary

128 code enciphers 128 ASCII characters and 100 numbers from 00 to 99, even some special characters used by characters A, B and C, the character encode by each of characters as following:

Characters A: ASCII character 00H to 5FH

Character B: ASCII character 20H to 7FH

Character C: the 100 numbers from 00-99

128-code also can encode the following special character:

SHIFT character

“SHIFT” enable to change barcode code in the first character A to B, or from B to A, it also can back to used characters under using SHIFT from the second character.

“SHIFT”character only can exchange between character A and B, it cannot function the current code character back to input or output characters C.

Character choice by character sets （CODE A、CODE B、 CODE C）

These characters can exchange the back code character to characters A, B and C.

The functional character（FNC1、 FNC2、FNC3、FNC4）

The usage for these functional characters depends on the applied software. In characters C, only FNC1 can be used.

B．2．1  128-code character sets

Character sets A

| Character | Sending data |  | Character | Sending data |  | Character | Sending data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  | Hex | Decimal |
| NULL SOH STX ETX EOT ENQ ACK BEL BS HT LF VT FF CR SO SI DLE DC1 DC2 DC3 DC4 NAK SYN ETB CAN EM SUB ESC FS GS RS US SP ! " # $ % & ' | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24 25 26 27 | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 | ( ) * + , - . / 0 1 2 3 4 5 6 7 8 9 : ; < = > ? @ A B C D E F G H I J K L M N O | 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F | 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 | P Q R S T U V W X Y Z [ \ ] ^ _ FNC1 FNC2 FNC3 FNC4 SHIFT CODEB CODEC | 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 7B,31 7B,32 7B,33 7B,34 7B,53 7B,42 7B,43 | 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 123,49 123,50 123,51 123,52 123,83 123,66 123,67 |

Character sets B

| Character | Sending data |  | Character | Sending data |  | Character | Sending data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  | Hex | Decimal |
| SP ! " # $ % & ' ( ) * + , - . / 0 1 2 3 4 5 6 7 8 9 : ; < = > ? @ A B C D E F G | 20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 | 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 | H I J K L M N O P Q R S T U V W X Y Z [ \ ] ^ _ ` a b c d e f g h i j k l m n o | 48 49 4A 4B 4C 4D 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F | 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 | p q r s t u v w x y z { ¦ } — DEL FNC1 FNC2 FNC3 FNC4 SHIFT CODEA CODEC | 70 71 72 73 74 75 76 77 78 79 7A 7B,7B 7C 7D 7E 7F 7B,31 7B,32 7B,33 7B,34 7B,53 7B,41 7B,43 | 112 113 114 115 116 117 118 119 120 121 122 123,123 124 125 126 127 123,49 123,50 123,51 123,52 123,83 123,65 123,67 |

Character sets C

| Character | Sending data |  | Character | Sending data |  | Character | Sending data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  | Hex | Decimal |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24 25 26 27 | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 | 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 | 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F | 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 | 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 FNC1 CODEA CODEB | 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 7B,31 7B,41 7B,42 | 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 123,49 123,65 123,66 |

## Appendix C  USB interface supplementary instruction

To print data via USB port, the driver should be pre-installed in the printer, says RP58 or RP80 driver. Then the first parameter set to be printer itself name, the last parameter to be “3”, when invoke “open”. After open invoked, please use “”IsOpen” to ensure it works, and confirm the printer’s name correct, if it cannot work. Moreover, “lasterror” can be used to find out the error (Choose to use “USB00x” in printer property dialog).

Opening the printer’s port by driver, only “write” available, and “read, timeout” will be ignored.

PS. If a printer commits printing by driver, all the data will be sent to driver by SDK. The designated port will be set by driver. (It means, the printer supports parallel interface and USB port etc.)